Cisco Router Forensics Checklist

Thomas Akin—July 15, 2002

The following checklist is to help you record the volatile information when performing router forensics. Printing out your recorded session and having both you and a witness sign it is to help make sure that if the evidence is ever entered into testimony, it is much harder to refute than if you simply had an electronic copy. Digital signatures can perform the same thing, but are much harder for judges and juries to understand.

- Connect to the console port. This is the least intrusive way to access the router and it doesn’t require network access that might warn your attacker if they are sniffing the network.

- Configure the terminal emulation software to record the entire session.

- Then login to the router and issue the following commands:

- enable – To enter enable mode.

- show clock detail – To record the routers current time and date

- In your notebook, record the exact time and date form a trusted source

- show version – Show what version IOS the router is running.

- show running-config – Show the configuration that is currently running in RAM

- show startup-config – Show the configuration that is set to run at start-up

- show reload – Determine if the router is schedule to auto-reboot anytime soon

- show ip route – Display the router’s routing tables

- show ip arp – Display the ARP tables

- show users – Show who is currently logged in

- show logging – Display the current logging buffer

- show ip interface – Display the configuration of all interfaces

- show tcp brief all – Display TCP connection endpoints

- show ip sockets – Show open network sockets

- show ip net translations verbose – Display the current NAT translations

- show ip cache flow – Display any NetFlows and their statistics

- show ip cef – Show entries in the Cisco Express Forwarding Table

- show snmp user – Display any SNMP version 3 users

- show snmp group – Display any SNMP version 3 groups

- show clock detail – Show the routers date and time again.

- Record the time from a trusted time source again.

- Disconnect from the router

- Stop recording your terminal session

- Print out the session you just recorded

- Write down the times the trusted time source on the print out.

- Sign and date the print out.

- Get a witness to sign and date the print out.

- Keep both the electronic copy and the hard copy in a secure location following proper chain of custody procedures.

Next, gather some information from the router remotely. If you do not have the password to access the router directly (i.e. the hacker changed them) then this may be the only information you are able to record about the router.

- Port scan the router with TCP, UDP, and RCP scan from a remote system.

- Use a trusted time source to record the time and date of the port scan.

- Print out the scan and write the time on the print out.

- If the router is running SNMP, get a copy of the current SNMP MIB tree. This can be done with a program such as snmpwalk (from NetSNMP http://net-snmp.sourceforge.net) For example

snmpwalk –v1 router.domain.com public

snmpwalk –v1 router.domain.com private

- From a trusted time source record the time of the SNMP walk.

- Print your findings and write the time on the print out.

- Sign and date both printouts.

- Get a witness to sign and date both printouts.

- Keep all copies in a secure location following proper chain of custody.